Impervious Surfaces: Review of Recent Literature

Christine Tilburg and Merryl Alber

Introduction

Urbanization and the introduction of impervious surface cover are becoming an increasingly important issue for Georgia coastal communities. The South Atlantic region has exhibited the largest rate of population growth in the country with a 58% increase between 1980 and 2003 (NOAA 2004). The U.S. Census Bureau (2000) projects that by 2025, 11 million additional people will live in the Carolinas and Georgia. Between 1992 and 1997 Georgia had the largest increase from undeveloped land to developed land in the Southeast with a 27.4% change towards urbanization (USDA, 2000). As more people move to the coast, areas that were undeveloped are quickly being altered into urbanized land. Urban growth, both inland and at the coastal zone, puts many stresses on the physical, chemical, and biological resources of the environment. One of the most obvious results of development is the introduction of impervious surface.

Impervious surface consists of manmade coverings of the natural landscape where water cannot soak through, and includes any hard surface like roof tops, asphalt, or concrete. Any increase in impervious surface has a direct effect on water storage and movement in the system. Figure 1 details the changes in the hydrologic system that occur as the amount of impervious surface increases. In an undeveloped or forested system, runoff composes only 10% of the hydrologic flow with the rest of the water either lost to evaporation or infiltrating into the ground. However, as the amount of impervious surface increases to 35-50%, runoff increases to 30% and far less water is stored via infiltration. Runoff presents a particularly important problem to sensitive tidal creeks and estuarine systems: it frequently contains high concentrations of nutrients such as nitrogen and phosphorus, as well as pollutants such as polycyclic aromatic hydrocarbons (PAHs) and fecal coliform, which can cause harm to freshwater and marine organisms (Schiff, 2002; Lee, 2005, Mallin, 2006).

Figure 1: Effect of impervious surface cover on hydrologic flows (Paul and Meyer, 2001).  Evapo-transpiration is water that evaporates back into the atmosphere; infiltration (whether shallow or deep) is water that percolates through the soil and becomes groundwater; runoff is water that flows over land, directly into surface water.

Evapo-transpiration 40%

Evapo-transpiration 30%

Evapo-transpiration 38%

Evapo-transpiration 35%

Runoff 10%

Runoff 55%

Runoff 30%

Runoff 20%

Forested

10-20% Imperviousness

35-50% Imperviousness

75-100% Imperviousness

Shallow Infiltration 25%

Shallow Infiltration 21%

Shallow Infiltration 20%

Shallow Infiltration 10%

Deep Infiltration 25%

Deep Infiltration 21%

Deep Infiltration 15%

Deep Infiltration 5%

Current Research

Since the late 1970’s, scientists have been attempting to quantify the biological and chemical effects of increases in impervious surfaces on the natural environment. Initial research focused on the impact of urbanization on freshwater streams: Klein (1979), for example, found a decline in species diversity when impervious surface covered more than 12% of the system. Similar studies have followed. Table 1 provides a summary of the most recent papers and the percentages of impervious area that accompanied changes in stream conditions.

One study of particular interest focused on the Etowah River basin in Georgia. Roy et al. (2003) researched the relationship between stream organisms and the amount of urbanization in the stream catchment area. The researchers also recorded water quality parameters such as total suspended solids, nutrients (nitrogen and phosphorus), dissolved oxygen, and total chlorophyll. Greater species diversity was found when stream water quality was good and reduced water quality was observed when the amount of urban land cover was >15%.

Table 1: Ecosystem Health and the Relationship to Impervious Surface

| Location | Stream Condition Marker | % Impervious Area where marked change occurred | Reference |
| --- | --- | --- | --- |
| Delaware | Decline in species diversity | 8-15% | Paul and Meyer, 2001 |
| Georgia | Decline in species diversity | 15% | Roy et al., 2003 |
| North Carolina | Water quality (fecal coliform) | >10% | Mallin et al., 2000 |
| South Carolina | Physical, chemical, and biological indicators | 10-20% | Holland et al., 2004 |
| Washington | Loss of aquatic system function | 10% | Booth and Jackson, 1997 |
| Wisconsin | Decline in species diversity | 8-12% | Stepenuck et al., 2002 Wang et al., 2000 |

There has been less research on the effect of impervious surface in coastal areas as compared to inland sites. However, there have been several recent studies of this type for the Carolina coasts (which share physical and biological attributes with the Georgia coast). Mallin et al. (2000) studied five watersheds in North Carolina with varying amounts of urbanization. The project focused on fecal coliform as an indicator of water quality since high fecal coliform concentrations are frequently cited when beaches and shellfish areas are closed. The study found that fecal coliform abundance was significantly correlated with watershed population (i.e. as population increased, fecal coliform abundance increased), particularly when impervious cover was greater than 10% (Mallin et al., 2000). The relationship between impervious surface and fecal coliform concentration was even more striking, and suggests that impervious surface is a very good predictor of fecal coliform (Mallin, 2006).

Holland et al. (2004) also sought to study the relationship of watershed urbanization and tidal creek health. This work focused on twenty-three tidal creeks in South Carolina that represented different levels of urbanization. They found that creeks with higher levels of impervious cover had measurable physical, chemical, and biological impairments compared with undeveloped areas.  High impervious surface correlated with high sediment contamination, coarser sediments, high fecal coliform levels, and different organisms. Changes occurred in both the physical and chemical characteristics of the creeks when impervious cover exceeded 10-20%. Detrimental biological changes occurred when impervious cover exceeded 20-30%.

There is also a small body of research on the effects of stormwater runoff on marine systems. Schiff et al. (2002) exposed sea urchins to stormwater diluted with various concentrations of seawater. At concentration of 6-12% stormwater, sea urchins exhibited a reduction in reproduction fertilization. The stormwater was analyzed for several chemical constituents, including nutrients and trace metals.  The researchers suggested that trace metals were responsible for the reproduction changes observed.

In two separate studies (one in SC and one in GA), Lee (2005; Lee et al., 2004) tested the effect that PAHs originating from stormwater runoff might have on grass shrimp. In both cases, he found that exposure to PAHs increased breakage in DNA and reduced embryonic development.  For the Georgia study, Lee studied PAHs and other chemical pollutants originating from runoff and retention ponds located in Chatham, Glynn and McIntosh counties near several Georgia estuaries.  Along with PAHs, Lee also reported elevated concentrations in the retention ponds of metals and pesticides while estuarine sediments had higher concentrations of copper, chromium, lead, zinc and nickel.

Conclusions

The studies described here demonstrate that increases in impervious surface and concomitant surface runoff routinely lead to impairments in water quality and biological communities in both freshwater and estuarine systems (see also Paul and Meyer, 2001). Although the results vary from study to study, most studies find that detrimental effects are observable when the amount of impervious surface increases past 10-15%, with biological changes frequently observed above 15% (Mallin et al., 2000; Roy et al., 2003; Holland et al., 2004). Booth and Jackson (1997) and Booth et al. (2002) suggest that changes that occur due to increases in impervious surface through watersheds are irreversible. Paul and Meyer (2001) argue that if stream restoration is possible, it will require scientists and planners to reintroduce or stabilize the physical, chemical, and biological systems of the impaired systems

References:

Booth, D.B. and C.R. Jackson (1997) Urbanization of aquatic systems – degradation thresholds, stormwater detention, and the limits of mitigation. Journal of the American Water Resources Association 22 (5): 1-20.

Booth, D.B., D. Hartley, and R. Jackson (2002) Forest Cover, Impervious-Surface Area, and the Mitigation of Stormwater Impacts. Journal of the American Water Resources Association 38: 835-845.

Holland, A.F., D.M. Sanger, C.P. Gawle, S.B. Lerberg, M.S. Santiago, G.H.M. Riekerk, L.E. Zimmerman, and G.I. Scott (2004) Linkages between tidal creek ecosystems and the landscape and demographic attributes of their watersheds. Journal of Experimental Marine Biology and Ecology 298: 151-178.

Klein, R.D. (1979) Urbanization and stream water quality impairment. Water Resources Bulletin 15: 948-963.

Lee, R.F., K.A. Maruya, and K. Bulski (2004) Exposure of grass shrimp to sediments receiving highway runoff: Effects on reproduction and DNA. Marine Environmental Research 58: 713-717.

Lee, R.F. (2005) Final Report for Cycle V Coastal Incentive Grant “Genototoxicants in Retention Ponds and Adjacent Estuaries in Urban/Suburban Sites in Coastal Georgia”. Skidaway Institute of Oceanography, 10 pages.

Mallin, M.A., K.E. Williams, E.C. Esham, and R.P. Lowe (2000) Effect of Human Development on Bacteriological Water Quality in Coastal Watersheds. Ecological Applications 10(4): 1047-1056.

Mallin, M.A. (2006) Wading in Waste. Scientific American June 2006 issue: 50-59.

National Oceanic and Atmospheric Administration (NOAA).  (2004)  Population Trends along the Coastal United States, 1980-2008, by Crossett, K.N., T.J. Culliton, P.C. Wiley and T.R. Goodspeed.  NOAA National Ocean Service Special Projects Office.  Silver Spring, MD, 47pages.

Paul, M.J. and J.L. Meyer (2001) Streams in the Urban Landscape. Annual Review of Ecology and Systematics 32: 333-365.

Roy, A.H., A.D. Rosemond, M.J. Paul, D.S. Leigh, and J.B. Wallace (2003) Stream macroinvertebrate response to catchment urbanization (Georgia, U.S.A.). Freshwater Biology 48: 329-346.

Schiff, K., S. Bay, and C. Stransky (2002) Characterization of stormwater toxicants from an urban watershed to freshwater and marine organisms. Urban Water 4: 215-227.

Stepenuck, K.F., R.L. Crunkilton, L.Z. Wang (2002) Impacts of urban land use on macroinvertebrate communities in southeastern Wisconsin streams. Journal of the American Water Resources Association 37: 1475-1487.

U.S. Department of Agriculture (2000) Summary Report: 1997 National Resources Inventory (revised December 2000), Natural Resources Conservation Service, Washington, D.C. and Statistical Laboratory, Iowa State University, Ames, Iowa, 89 pages.

U.S. Department of Commerce – Census Bureau (2005) Cumulative Estimates of Population Change for the United States and States, and for Puerto Rico and State Rankings: April 1, 2000 to July 1, 2005 (NST-EST2005-02) data provided by the Census Bureau webpage (http://www.census.gov/popest/states/NST-pop-chg.html) and accessed 2/17/05.